Step 1: Understand and Define the Problem (Analyze)

- Create a logic circuit that sounds an alarm when the car is in motion and either one of the front seatbelts is not engaged while a driver or passenger is present in the car, while the ignition is on.

- The DRIV or PASS sensors give off HIGH if the presence of a driver or a passenger is detected.

- The IGN is HIGH if the ignition is on.

- The ^BELTD or ^BELTP become HIGH if the driver or passenger seatbelt is not engaged

Conditions for the alarm to sound would be:

- The driver is present AND the belt is unfastened

Or

- The passenger is present AND the belt is unfastened

AND

- The ignition is on.

Step 2: Organize and Describe the Data

| Symbol | Meaning | Type | Active logic |
| --- | --- | --- | --- |
| DRIV | Indicates the presence of a driver | input | 1 |
| PASS | Indicates the presence of a passenger | Input | 1 |
| BELTD’ | Indicates the Driver’s seatbelt is not engaged | Input | 0 |
| BELTP’ | Indicates the Passenger’s seatbelt is not engaged | Input | 0 |
| IGN | Indicates the car ignition is on | Input | 1 |
| Alarm | Alarm signal | output | 0 |

Step 3.1: Plan the Solution (Design the Algorithm)

- Check IGN status

- Check if driv is seated and belt is not fastened

DRIV = 1 AND BELTD’=0

- Check if passenger is seated and belt fastened

PASS = 1 AND BELTP’ =0

- If either of the above is true, alarm is on (0).

- If both point 2 and 3 are FALSE, alarm is off.

3.2 TRUTH TABLE

| IGN | DRIV | PASS | BELTD’ | BELTP’ | Alarm |
| --- | --- | --- | --- | --- | --- |
| 1 | 0 | 0 | 0 | 0 | 1 |
| 1 | 0 | 0 | 0 | 1 | 1 |
| 1 | 0 | 0 | 1 | 0 | 1 |
| 1 | 0 | 0 | 1 | 1 | 1 |
| 1 | 0 | 1 | 0 | 0 | 1 |
| 1 | 0 | 1 | 0 | 1 | 1 |
| 1 | 0 | 1 | 1 | 0 | 1 |
| 1 | 0 | 1 | 1 | 1 | 1 |
| 1 | 1 | 0 | 0 | 0 | 0, Alarm is ON |
| 1 | 1 | 0 | 0 | 1 | 0, Alarm is ON |
| 1 | 1 | 0 | 1 | 0 | 1, |
| 1 | 1 | 0 | 1 | 1 | 1 |
| 1 | 1 | 1 | 0 | 0 | 0, Alarm is ON |
| 1 | 1 | 1 | 0 | 1 | 0, Alarm is ON |
| 1 | 1 | 1 | 1 | 0 | 0, Alarm is ON |
| 1 | 1 | 1 | 1 | 1 | 1, |

3.3 Flow Chart

3.4 Boolean expression:

For the Alarm to be ON:

IGN.DRIV.PASS.BELTD.BELTP

IGN.DRIV.PASS. BELTD.BELTP

IGN.DRIV.PASS. BELTD. BELTP

IGN.DRIV.PASS. BELTD.BELTP

IGN.DRIV.PASS.BELTD. BELTP

ALARM =  IGN.DRIV.PASS.BELTD.BELTP+ IGN.DRIV.PASS. BELTD.BELTP + IGN.DRIV.PASS. BELTD. BELTP +  IGN.DRIV.PASS. BELTD.BELTP + IGN.DRIV.PASS.BELTD. BELTP

3.5 Pseudocode:

START

GET IGN, DRIV, PASS, BELTD, BELTP

IF  IGN.DRIV.PASS.BELTD.BELTP+ IGN.DRIV.PASS. BELTD.BELTP + IGN.DRIV.PASS. BELTD. BELTP +  IGN.DRIV.PASS. BELTD.BELTP + IGN.DRIV.PASS.BELTD. BELTP

Then ALARM = 0 # ON

Else ALARM = 1 #OFF

END

Step 4: Verify and debug

Input: DRIV.^PASS.^BELTD.^BELTP

Output: 0

Conclusion: The alarm will turn on as the driver is present in the car and the belt is unfastened while the ignition is on.

Input: DRIV.^PASS.BELTD.^BELTP

Output: 0

Conclusion: The alarm is off as the driver’s seatbelt is fastened while the ignition is on.

Input: ^DRIV.PASS.^BELTD.^BELTP

Output: 0

Conclusion: The alarm is off even though the Passenger is present and has an unfastened seatbelt with ignition on because the driver is not present in the car.

Input: DRIV.PASS.^BELTD.^BELTP

Output: 0

Conclusion: The alarm is on as both the driver and the passenger are present in the car with an unfastened seatbelt while the ignition is on.